Butte County Fire Department

## VOLUNTEER ENGINEER

PERFORMANCE EXAM:   FOREWARD HOSELAY

Volunteer Engineer: _______________________        VFC:_______________

### Apparatus Used: ____________________ Date of exam:_________________

PASS / FAIL

| 1. Mount tailboard facing hose bed |  |
| --- | --- |
| 2. Grasp hose in hose bed (3” or 5” bundle) |  |
| 3. Step down from the tailboard facing direction of travel |  |
| 4. Pull enough hose to reach beyond the hydrant |  |
| 5. Remove the hydrant wrench and any appliances from the bundle |  |
| 6. Anchor hose with both hands, thumbs up at edge of hose |  |
| 7. Give the command to “lay line” loudly, motion w/ hands |  |
| 8. When 100’ of hose is on the ground, ground hose |  |
| 9. Remove hydrant cap and flush slowly. Close hydrant |  |
| 10. Connect hose supply line to hydrant and wait for command to charge the hydrant. When given charge slowly and w/ control. |  |

My signature below indicates the individual listed above has successfully completed the above performance exam.

| Proctor (career company officer signature) |  | Print Name |
| --- | --- | --- |

After the performance exam has the approval signature, please send it to the Training & Safety Bureau where it will be kept in the employees training file.