Butte County Fire Department

## VOLUNTEER ENGINEER

PERFORMANCE EXAM:    PUMP FROM DRAFT

VOLUNTEER ENGINEER: _______________________COMPANY: ______

APPARATUS: _________________________________ DATE :___________

PASS / FAIL

| 1. Spot apparatus next to water source (hard surface) |  |
| --- | --- |
| 2. Set chock blocks |  |
| 3. Connect hard section to inlet suction valve. |  |
| 4. Place foot on brake and start engine. |  |
| 5. Close tank suction valve (all other valves except inlet suction) |  |
| 6. Open suction inlet suction valve |  |
| 7. Engage primer for no longer than 30 seconds, turn off primer |  |
| 8. Return to cab and engage pump |  |
| 9. Adjust pump panel pressure to 150 PSI +/- 20 PSI |  |
| 10. Loudly state “water coming” (attack line attached to discharge) |  |
| 11. Slowly open discharge valve completely |  |

My signature below indicates the individual listed above has successfully completed the above performance exam.

| Proctor (career company officer signature) |  | Print Name |
| --- | --- | --- |

After the performance exam has the approval signature, please send the form to the Training & Safety Bureau where it will be kept in the employees training file.